## Department of Public Works Comments

CASE NAME: HCSB - Beaverdam

TRACKIT PROJECT #: CUP2024-00004

PLEASE MARK THE APPROPRIATE BOX BELOW:

No Comment

Comments Below

DATE:	   02/13/20234	REVIEWED BY:   Michael J Dieter

No comments on the CUP.

1.	Wetlands delineation, evaluation of CBPA and extent of RPA required. Water quality impact assessment certification required along with associated supporting information.

2.	Additional development subject to runoff reduction and channel protection.

3.	With the large increase in impervious area, it is likely that the onsite detention of stormwater will be required.

4.	If development is 5 acres or more or 10lb or more of phos load, 75% of nutrient reduction must be met on site.

5.	Analysis of downstream manmade and natural systems to a point where the site is less than 1% of drainage area will likely be required.

6.	Downstream pond will require that the site provide detention to predevelopment 2, 10, and 100 year storms.