## Department of Public Works Comments

CASE NAME: LONEOAK LAND CO., L.L.C.

TRACKIT PROJECT #: REZ2023-00030

PLEASE MARK THE APPROPRIATE BOX BELOW:

No Comment

Comments Below

DATE:	   08/16/2023		REVIEWED BY:   Michael J Dieter

No comments on the Rezoning.

- Additional development subject to runoff reduction and channel protection.

- With the large increase in impervious area, it is likely that the onsite detention of stormwater will be required.

- If development is 5 acres or more or 10lb or more of phos load, 75% of nutrient reduction must be met on site.

- Wetlands delineation, evaluation of CBPA and extent of RPA required. Water quality impact assessment certification required along with associated supporting information.

- Analysis of downstream manmade and natural systems to a point where the site is less than 1% of drainage area will likely be required.