## Department of Public Works Comments

CASE NAME: Marianne Rae Minton

TRACKIT PROJECT #: REZ2023-00046

PLEASE MARK THE APPROPRIATE BOX BELOW:

No Comment

Comments Below

DATE:	   01/08/2024		REVIEWED BY:   Michael J Dieter

No comments on rezoning.  Comments are for development plans.

1.	Additional development subject to runoff reduction and channel protection.

2.	With the large increase in impervious area, it is likely that the onsite detention of stormwater will be required.

3.	If development is 5 acres or more or 10lb or more of phos load, 75% of nutrient reduction must be met on site.

4.	Wetlands delineation, evaluation of CBPA and extent of RPA required. Water quality impact assessment certification required along with associated supporting information.

5.	Analysis of downstream manmade and natural systems to a point where the site is less than 1% of drainage area will likely be required.

6.	Wetland per national wetland inventory occupy substantial portions of the lot.  A site specific delineation of the extent of wetlands and approval of any impacts will be required prior to construction plan approval.

7.  Provide for BMP access