# REZONING

## Department of Public Works Comments

CASE NAME: Westhaven Properties LLC

TRACKIT PROJECT #: REZ2023-00024

PLEASE MARK THE APPROPRIATE BOX BELOW:

No Comment

Comments Below

DATE:	   07/13/2023					REVIEWED BY:   MJD

No comments on rezoning

Comments for site development:

- Additional development subject to runoff reduction and channel protection.

- If development of 5 acres or more and 10 lbs or more of nutrient reduction, a 75% reduction in nutrient loading must be achieved on-site.

- Must show that the development is less than 1% of area or flow at discharge point to the natural channel, or must meet energy balance on-site.

- Must obtain appropriate permits from US Army Corps/DEQ for any work on the lake.

- No FEMA floodplain onsite now or in proposed update 2021.

- A portion of the pond onsite is within the dam breach zone for Pebble Creek Pond Dam Safety Permit #08532.  Inundation zone mapping must be shown on plans.

- Per Hanover Ordinances (Sect 10-40), redevelopment shall be permitted in the RPA only if there is no increase in the amount of impervious cover and no further encroachment within the RPA, and shall conform to all applicable federal, state and county erosion and sediment control and stormwater management laws and regulations.  Restoration of the buffer may be required in some areas per CBPA buffer mitigation manual.   Any encroachment including new parking, buildings or associated grading and erosion controls will not be allowed.

- Paths in the RPA (section 10-45d) shown on drawing may require grading at the locations where the paths are currently shown.  Pulling paths back from the edge of the water will provide a path, but minimal grading would be required.  Grading should be minimum necessary to meet the intended purpose.

- Pond must be shown to meet Hanover County Drainage Design Handbook requirements.  Dam safety regulations may, but likely don’t apply.  Please check applicability.

- Detention will be required to show no impact to downstream pond at Cherrydale.