## Department of Public Works Comments

CASE NAME: MARCHETTIE PROPERTIES, ET AL.

TRACKIT PROJECT #: REZ2025-00023

PLEASE MARK THE APPROPRIATE BOX BELOW:

No Comment

Comments Below

DATE:	   10/15/2025		REVIEWED BY:   MJD

No comments on zoning actions

Comments on future development.

- Wetlands delineation, evaluation of CBPA and extent of RPA required. Water quality impact assessment certification required along with associated supporting information.

- Additional development subject to state stormwater requirements and meet runoff reduction, channel protection, and flood protection requirements.

- With the large increase in impervious area, it is likely that the onsite detention of stormwater will be required.

- If development is 5 acres or more or 10lb or more of phos load, 75% of nutrient reduction must be met on site.

- Analysis of downstream manmade and natural systems to a point where the site is less than 1% of drainage area will likely be required.

- Holtzgraph Dam #085017 located onsite is regulated by DCR.  Dam must be compliant with dam safety requirements if it remains.

- Onsite ponds must meet the requirements of the Hanover County Drainage Design Handbook.    Holtzgraph Dam #085017  must be current with state permitting and meet Hanover County requirements.